20. Cableado euroconector

Lado largo:    2     NF IN R Const. (Audio)

4     Audio masa

6     NF IN R. Const. (Audio)

8     12V continua

10    Data 2

12    Data 1

14    Data masa

16    Switch video interno/externo

18    Switch masa

20    vídeo in (el que a ti te interesa)

Lado corto:    1     NF OUT R Const. (Audio)

3     NF OUT L Const. (Audio)

5     B extern masa

7     vídeo B extern

9     G extern masa

11    vídeo G extern

13    R extern masa

15    vídeo R extern

17    vídeo tierra

19    vídeo out

Los pines 1 y 2 están en lado más cuadrado, y los pines 19 y 20 en el más trapezoidal.

Para más información lo tienes en:

http://www.ntua.gr/electronics/hwb/co_Scart.html

EUROCONECTOR (SCART)

(At the video/TV)		(At the cable)

21 PIN SCART FEMALE at the Video/TV.
21 PIN SCART MALE at the Cable.

| Pin | Name | Description | Signal Level | Impedance |
| --- | --- | --- | --- | --- |
| 1 | AOR | Audio Out Right | 0.5 V rms | <1k ohm |
| 2 | AIR | Audio In Right | 0.5 V rms | >10k ohm |
| 3 | AOL | Audio Out Left + Mono | 0.5 V rms | <1k ohm |
| 4 | AGND | Audio Ground |  |  |
| 5 | B GND | RGB Blue Ground |  |  |
| 6 | AIL | Audio In Left + Mono | 0.5 V rms | >10k ohm |
| 7 | B | RGB Blue In | 0.7 V | 75 ohm |
| 8 | SWTCH | Audio/RGB switch / 16:9 |  |  |
| 9 | G GND | RGB Green Ground |  |  |
| 10 | CLKOUT | Data 2: Clockpulse Out (Unavailble ??) |  |  |
| 11 | G | RGB Green In | 0.7 V | 75 ohm |
| 12 | DATA | Data 1: Data Out (Unavailble ??) |  |  |
| 13 | R GND | RGB Red Ground |  |  |
| 14 | DATAGND | Data Ground |  |  |
| 15 | R | RGB Red In / Chrominance | 0.7 V (Chrom.: 0.3 V burst) | 75 ohm |
| 16 | BLNK | Blanking Signal | 1-3 V=RGB, 0-0.4 V=Composite | 75 ohm |
| 17 | VGND | Composite Video Ground |  |  |
| 18 | BLNKGND | Blanking Signal Ground |  |  |
| 19 | VOUT | Composite Video Out | 1 V | 75 ohm |
| 20 | VIN | Composite Video In / Luminance | 1 V | 75 ohm |
| 21 | SHIELD | Ground/Shield (Chassis) |  |  |